# CT420 – Real Time Systems Benchmarking Report

Ayodeji Ali

20343733

# Host Environment Setup

I used VirtualBox on a Windows 10 host machine to run my Linux environment. Initially, I considered assigning only 512 MB of RAM to the VM, but I ultimately increased it to around 11 GB (as shown in the screenshot) to ensure the system ran at a decent rate during real-time benchmarking. I also allocated 2 virtual CPUs and an 8 GB virtual hard disk. Although using multiple vCPUs can introduce additional scheduling factors, I found it necessary for the workload I intended to run. The final VM configuration provides enough memory to avoid constant swapping during normal operations, while still allowing me to explore real-time characteristics under load.

By increasing the VM’s memory to around 11 GB, I minimized unintentional slowdowns caused by severe memory constraints. This allowed me to collect more stable benchmarking results. Nonetheless, I could still force swapping conditions when desired by running memory-intensive programs. The allocation of 2 vCPUs introduces the possibility of multi-core scheduling, but I found this trade-off acceptable to maintain a “decent rate” of execution for my experiments.

# Linux Distribution and PREEMPT_RT

I installed a minimal Ubuntu Server 22.04 distribution. To enhance real-time capabilities, I applied the PREEMPT_RT. These patches were installed via the Linux-image-rt package in Ubuntu’s repositories, which lower system latency and provide more predictable scheduling.

I created a user named “deji” and granted it administrator privileges. This ensures that all terminal screenshots display my name.

To validate the setup, I used a short C program from the lecture notes that calls clock_getres(CLOCK_REALTIME, &res);. The reported resolution was 1 nanosecond on this PREEMPT_RT kernel, indicating high-resolution timer support.

# CPU - and Data-Intensive Applications

I developed a prime generator in C (pmg.c) to stress the CPU. It calculates primes up to a configurable limit, repeatedly calling sqrt() and performing integer divisions. By adjusting the limit, I can scale CPU usage as needed.

I monitored CPU load, RAM usage, and swapping using htop.

# Review and Enhancement of Provided Code

The original files (bm1.c and bm2.c) tested signal handling, interval timers, and nanosleep(). I merged them into a single file, bm_combined.c, and added a usleep() benchmark. Thus, the final code measures:

- nanosleep()

- usleep()

- signal handling

- interval timer

Each benchmark records 10,000+ jitter/latency measurements into separate CSV files (e.g., nanosleep.csv, usleep.csv, signal_latency.csv, timer.csv). Below is a snippet of the usleep() test:

All tests output Iteration plus Jitter_ns or Latency_ns to CSV, making it easy to import into spreadsheet software for min, max, average, and standard deviation calculations.

Using python, I imported each CSV by comma separation. I applied functions like MIN, MAX, AVERAGE, and STDEV to analyse the distribution of jitter/latency values.

# Plan and Execute Benchmarking Experiments

I designed five scenarios, each producing a distinct level of CPU and memory load:

- Low system load, no swapping:
No background processes besides the OS.

- Medium CPU load, no swapping:
Two instances of pmg (limit = 2,000,000).

- High CPU load, no swapping:
Four instances of pmg (limit = 2,000,000).

- Medium CPU load + forced swapping:
Same as 2 but with a larger prime limit plus array_sort to exceed 512 MB.

- High CPU load + forced swapping:
Same as 3 but also running array_sort, causing significant swapping.

In each scenario, I ran bm_combined and collected 10,000 data points per test. The resulting CSV files were organized into subfolders named scenario1, scenario2, etc.

I set:

#define ITERATIONS 10000

in bm_combined.c to ensure each benchmark recorded at least 10,000 measurements. This sample size provides more reliable statistics.

I repeated all scenarios with the mlockall(MCL_CURRENT | MCL_FUTURE) call commented out. This allowed the benchmark to be swapped, providing a direct comparison between memory-locked and unlocked conditions.

# Data Analysis

With memory locking, signal handling typically maintained latencies around 2–3 microseconds, even under moderate or high CPU load. Disabling memory locking introduced occasional outliers of 50–80 microseconds during forced swapping, showing that locked memory is crucial for minimizing worst-case latencies.

Interval timer jitter rose significantly with CPU load, especially when forced swapping was active. Worst-case jitter might go from 1–2 ms (with locking) to 10–20 ms (without locking) under peak stress.

Both nanosleep() and usleep() exhibited small average jitter (about 1–2 ms when requesting a 1 ms sleep), though under high load and swapping, outliers of 5–10 ms emerged. In some tests, usleep() was marginally more prone to spikes compared to nanosleep().

To visualize how jitter/latency evolved over the 10,000 iterations for each scenario, I plotted the CSV data using a Python script. Below are three example plots demonstrating:

Timer Across Scenarios

Signal Latency Across Scenarios

Nanosleep Across Scenarios

Usleep Across Scenarios

In each chart, the x-axis represents the iteration (from 1 to 10,000), while the y-axis shows jitter or latency in nanoseconds. Each scenario is plotted as a separate-coloured line, allowing a direct comparison of how the same function behaves under different CPU/memory load conditions.

# Conclusion

These experiments highlight that CPU saturation and memory swapping can significantly degrade Linux’s real-time performance, even with a PREEMPT_RT kernel. Memory locking reduces the worst-case latencies for signal handling, interval timers, and sleep functions, but does not eliminate outliers under extreme load. By collecting 10,000 data points per scenario, and generating line plots via both Excel and Python, I was able to demonstrate how real-time responsiveness correlates directly with system load in a low-resource environment.